PocketLab BLE Write Issue — Problem and Fix

Symptom

The React Native app connected to the ESP32 successfully and could:

- scan

- connect

- discover services

- read characteristics

- subscribe to notifications

But writes appeared to succeed without actually reaching the ESP32.

App logs showed:

[BLE] Command write accepted

[BLE] Command RX read-back value: "EMPTY"

ESP32 logs showed:

READ: EMPTY

The ESP32 never entered onWrite().

What was ruled out

The issue was tested with:

- NimBLE-Arduino

- the ESP32 built-in Bluedroid BLE library

- ESP32 core 3.3.10

- ESP32 core 3.3.6

- fresh UUIDs

- write with response

- write without response

- a minimal one-characteristic firmware

- Android nRF Connect

- Windows Bleak

Windows Bleak and nRF Connect eventually proved that the ESP32 firmware and characteristic were working correctly:

WRITE length=4 value=PING

READ length=4 value=PING

So the problem was not the ESP32, NimBLE, Bluedroid, UUIDs, or Android BLE itself.

Actual cause

The React Native BLE client was not reliably writing and reading through the same authoritative connected Device object.

The normal helper path looked correct and reported success, but it could use a stale or mismatched connection reference after connection or reconnection activity.

Fix

Store the connected device returned by:

await device.connect()

and then:

await connectedDevice.discoverAllServicesAndCharacteristics()

Use that exact stored Device object for both the write and the read-back.

The successful diagnostic method was:

public async directDeviceWriteTest(command = "PING"): Promise<string> {

const device = this.requireConnectedDevice();

const encodedValue = Buffer.from(command, "utf8").toString("base64");

const connected = await device.isConnected();

if (!connected) {

throw new Error("Stored BLE device is not connected.");

}

await device.writeCharacteristicWithResponseForService(

POCKETLAB_SERVICE_UUID,

COMMAND_RX_UUID,

encodedValue

);

await delay(500);

const readResult = await device.readCharacteristicForService(

POCKETLAB_SERVICE_UUID,

COMMAND_RX_UUID

);

return readResult.value

? Buffer.from(readResult.value, "base64").toString("utf8")

: "";

}

The provider called it directly:

const testWrite = useCallback(async (): Promise<void> => {

const commandValue =

await bleDiagnostic.directDeviceWriteTest("PING");

if (commandValue !== "PING") {

throw new Error(

`Direct write failed. Read-back was "${commandValue}".`

);

}

}, []);

Successful result

React Native write:

PING

ESP32 output:

WRITE length=4 value=PING

READ length=4 value=PING

Rule to remember

For react-native-ble-plx, after connecting and discovering services, keep one authoritative connected Device object and use that same object for:

- writes

- reads

- notifications

- connection checks

Avoid mixing:

- scanned Device objects

- older device references

- manager-level operations

- device objects from earlier connection generations

especially after reconnects.

Quick diagnostic checklist

When BLE writes appear successful but the peripheral does not receive them:

- Confirm the peripheral works with another client such as Bleak or nRF Connect.

- Verify the write is sent as Base64.

- Use write with response first.

- Call device.isConnected().

- Write and read through the same stored Device object.

- Log:

  - device ID

  - service UUID

  - characteristic UUID

  - connection generation

  - encoded value

  - decoded read-back

- Watch for stale disconnect callbacks replacing or clearing the active device reference