X9C103 Solid-State Potentiometer 10kΩ

https://www.amazon.com/dp/B07Y23SY7L?ref=ppx_yo2ov_dt_b_fed_asin_title
https://www.renesas.com/en/products/x9c103?utm_source=renesas&utm_medium=datasheet&utm_campaign=x9c102-103-104-503-ds-description

- Solid-State Potentiometer

- Three-Wire Serial Interface

- 100 Wiper Tap Points

- Wiper Position Stored in Non-volatile Memory and Recalled on Power-up

- 99 Resistive Elements

- Temperature Compensated

- End-to-End Resistance, ±20%

- Terminal Voltages, ±5V

- Low Power CMOS

- VCC = 5V

- Active Current, 3mA max.

- Standby Current, 750µA max.

- High Reliability

- Endurance, 100,000 Data Changes per Bit

- Register Data Retention, 100 years

- X9C102 = 1kΩ

- X9C103 = 10kΩ

- X9C503 = 50kΩ

- X9C104 = 100kΩ

- Packages

- 8 Ld SOIC

- 8 Ld PDIP

- Pb-Free Available (RoHS Compliant)

Module control connections

| Module pin | Function | ESP32 connection |
| --- | --- | --- |
| VCC | Module power | 5 V |
| CS | Chip select | ESP32 GPIO |
| INC | Wiper step clock | ESP32 GPIO |
| U/D | Direction | ESP32 GPIO |
| GND | Ground | Common ground |
| VH | Potentiometer high terminal | AD9833 conditioned signal |
| VW | Wiper | OP484 input |
| VL | Potentiometer low terminal | Buffered 2.5 V reference |

The X9C103 is controlled by pulses rather than SPI or I²C:

- CS enables the device.

- U/D selects the direction.

- Each falling edge on INC moves the wiper one position.

- It has 100 wiper positions.

- The position can be stored in nonvolatile memory.

The X9C103 has a nominal 10 kΩ end-to-end resistance, ±20% tolerance, and approximately 40 Ω typical wiper resistance. Its analog terminals support signals within the needed range for this circuit. Renesas X9C103 datasheet

The 3.3 V ESP32 control outputs should be compatible with its digital control inputs while the module operates from 5 V, but all grounds must be connected.

Important limitations

This is a useful prototype part, but probably not the ideal final amplitude-control component:

- Only 100 positions, giving roughly 1% amplitude increments

- No direct position readback

- The ESP32 must track the wiper position

- On startup, firmware may need to “home” it by sending at least 99 downward pulses

- ±20% resistance tolerance

- Limited nonvolatile-memory write endurance

- Switching between taps can introduce a small amplitude step or transient

- Signal bandwidth and distortion should be measured experimentally

Avoid saving the position to nonvolatile memory every time the user moves the amplitude slider. Its memory is rated for a finite number of writes. For PocketLab, I would home the wiper to zero at startup and maintain its position in firmware.