PocketLab Function Generator

# X9C Wiper Noise and Filter Evaluation

Engineering Test Report  •  17 July 2026

| Test configuration | Key outcome |
| --- | --- |
| Bench-powered 4.75 V analog circuit; midpoint X9C setting; 1 kHz–200 kHz sine tests | 100 pF wiper-reference capacitor gave the best tested noise suppression, with ~11% amplitude loss at 200 kHz |

# 1. Objective

Determine the source of the thick, distorted waveform observed at intermediate X9C103 wiper positions, evaluate power-supply and local filtering changes, and select a practical capacitor value that prioritizes noise reduction while preserving usable bandwidth.

# 2. Hardware and Test Configuration

| Item | Configuration |
| --- | --- |
| Signal chain | AD9833 → conditioned signal → X9C103 → OP484 output stage |
| Offset source | AD5626 DAC buffered by OP484; no separate VBIAS divider |
| X9C reference | VL connected to OP484 pin 7 (buffered DAC offset) |
| Compensation capacitor | Installed between X9C VW and OP484 pin 7 |
| OP484 gain network | Rf = 62.2 kΩ, Rg = 10.00 kΩ; nominal gain = 7.22 V/V |
| Analog supply | 4.75 V bench supply; 10 µF + 47 µF bulk capacitance and local 100 nF bypassing |
| Instrumentation | Digilent Discovery 3 / WaveForms oscilloscope, logic analyzer, and FFT |
| Representative state | SINE, offset ≈ 2.30 V; X9C at an intermediate wiper position |

# 3. Diagnostic Findings

X9C digital control was correct: CS remained low during the update, U/D was stable, and 31 INC pulses were observed at roughly 200 µs per step.

The distortion was strongest at intermediate wiper positions and largely absent at the endpoints, consistent with the higher Thevenin impedance of the wiper near midscale.

FFT measurements revealed periodic interference near 1.05 MHz and harmonics near 2.10, 3.15, and 4.20 MHz. These components were coupled into the high-impedance wiper node.

Changing from laptop USB power to a bench supply substantially reduced long-timescale supply droop. Added bulk capacitance helped, but did not by itself remove wiper-node pickup.

The OP484 gain stage measured approximately 7.204 V/V using AC RMS measurements, closely matching the 7.22 V/V design value.

# 4. Filter Comparison

At approximately 1 kHz and the same intermediate wiper position, the residual non-sinusoidal thickness was estimated by subtracting the sine-derived peak-to-peak value from the measured peak-to-peak value.

| Condition | Measured p-p | Sine-derived p-p | Excess thickness | Interpretation |
| --- | --- | --- | --- | --- |
| No capacitor | 297.4 mV | 244.1 mV | 53.3 mV | Strong pickup/distortion |
| 68 pF | 270.6 mV | 256.8 mV | 13.8 mV | Major improvement |
| 100 pF | 268.88 mV | 262.36 mV | 6.52 mV | Best tested suppression |

The 100 pF capacitor reduced the remaining excess thickness by approximately 53% relative to 68 pF, and by approximately 88% relative to the unfiltered midpoint measurement.

# 5. Frequency-Response Results

C1 measured X9C VW and C2 measured X9C VH. Wiper ratio = C1 AC RMS ÷ C2 AC RMS.

| Capacitor | Frequency | VW AC RMS | VH AC RMS | Wiper ratio | Change from 1 kHz |
| --- | --- | --- | --- | --- | --- |
| 68 pF | 1 kHz | — | — | 0.45134 | Reference |
| 68 pF | 200 kHz | — | — | 0.41579 | −7.88% (−0.71 dB) |
| 100 pF | 1 kHz | 92.845 mV | 205.84 mV | 0.45106 | Reference |
| 100 pF | 10 kHz | 93.123 mV | 206.56 mV | 0.45084 | −0.05% |
| 100 pF | 50 kHz | 92.482 mV | 207.19 mV | 0.44636 | −1.04% |
| 100 pF | 100 kHz | 89.746 mV | 206.79 mV | 0.43398 | −3.79% |
| 100 pF | 200 kHz | 82.451 mV | 206.11 mV | 0.40004 | −11.31% (−1.04 dB) |

# 6. Interpretation

The original high-frequency waveform degradation was not primarily an AD9833 waveform-generation limit. It was dominated by interference coupled into the X9C wiper while the wiper presented a high source impedance. The 100 pF capacitor provides a lower-impedance high-frequency path to the buffered offset reference, materially cleaning the waveform.

The tradeoff is predictable frequency-dependent attenuation. With 100 pF installed, the midpoint wiper ratio remained essentially unchanged through 10 kHz, fell about 3.8% by 100 kHz, and fell about 11.3% by 200 kHz. The present 200 kHz waveform is smooth and symmetric compared with the earlier multi-lobed, stair-stepped result.

# 7. Conclusions

Keep the 100 pF capacitor from X9C VW to OP484 pin 7 as the provisional hardware configuration.

Retain the bench-supply and local-decoupling improvements; power integrity and wiper filtering address different parts of the observed noise.

Treat the remaining amplitude roll-off as a calibration problem rather than removing the filter and reintroducing distortion.

Do not assume a single amplitude scale factor: correction will depend on both frequency and X9C wiper position.

# 8. Recommended Next Steps

Finalize the shorter-lead breadboard layout with the 100 pF capacitor and the current bypass/bulk capacitors installed.

Repeat the formal frequency sweep on the final physical layout, including 1, 10, 50, 100, 150, and 200 kHz checkpoints.

Build a two-dimensional calibration table across frequency and X9C positions (suggested positions: 0, 4, 8, 12, 16, 20, 24, 28, and 31).

Use interpolation in firmware to compensate amplitude attenuation while enforcing output-envelope and maximum-headroom limits.

Measure phase shift and harmonic distortion separately; amplitude correction cannot compensate phase lag or residual waveform-shape error.

Revalidate amplitude/offset envelope limits after calibration, especially at maximum amplitude and near the output rails.

# 9. Test Limitations

Measurements were made on a solderless breadboard and are sensitive to probe placement, lead length, grounding, X9C position, and acquisition settings. Values should therefore be treated as engineering characterization rather than final production specifications. The 100 pF decision is provisional until the rebuilt layout is swept under the same measurement conditions.